智能硬件设计基础上机报告（4）

学院（系）：    求实书院（两人/单人每组）

姓     名：                   学号：

姓     名：                   学号：

成     绩：

一、实验目的和要求

1 外部传感器的信息读取交互

2  BH1750FVI的数据读取问题

3  CW2015的数据读取问题

二、实验原理和内容

1 安装HiSparkStudio、下载并安装WS63 SDK

该步骤已经在第一次实验中进行完毕，此处不再介绍相关内容。

2 完成BH1750FVI的SDK的编写过程

该步骤主要包括如下部分。

(1) BH1750FVI的“.h”文件

#include "pinctrl.h"

#include "common_def.h"

#include "soc_osal.h"

#include "gpio.h"

#include "hal_gpio.h"

#include "app_init.h"

#define BH1750_ADDR 0x46

#define BH1750_POWER_OFF 0x00

#define BH1750_POWER_ON 0x01

#define BH1750_MODULE_RESET 0x07

#define	BH1750_CONTINUE_H_MODE 0x10

#define BH1750_CONTINUE_H_MODE2 0x11

#define BH1750_CONTINUE_L_MODE 0x13

#define BH1750_ONE_TIME_H_MODE 0x20

#define BH1750_ONE_TIME_H_MODE2 0x21

#define BH1750_ONE_TIME_L_MODE 0x23

#define MEASURE_MODE BH1750_CONTINUE_H_MODE

#define BH1750_RES 1

void bh1750_init(void);

uint16_t bh1750_GetLightIntensity(void);

(2) BH1750FVI的“.c”文件

#include "pinctrl.h"

#include "common_def.h"

#include "soc_osal.h"

#include "gpio.h"

#include "hal_gpio.h"

#include "app_init.h"

#include "bh1750.h"

#include "i2c.h"

#define I2C_MASTER_BUS_ID 1

uint16_t bh1750_buff_H = 0;

uint16_t bh1750_buff_L = 0;

(3) BH1750FVI的“.c”文件的主要参数

void bh1750_SendCMD(uint8_t cmd){

uint8_t buffer[] = {cmd};

i2c_data_t data = {0};

data.send_buf = buffer;

data.send_len = sizeof(buffer);

errcode_t ret = uapi_i2c_master_write(I2C_MASTER_BUS_ID,

BH1750_ADDR >> 1, &data);

if (ret != 0) {

printf("BH1750:I2cWriteCMD(%02X) failed, %0X!\n", cmd, ret);

return ;

}

}

void bh1750_ReadData(void){

uint8_t buffer[2] = {0};

i2c_data_t data;

data.receive_len = sizeof(buffer);

data.receive_buf = buffer;

errcode_t ret = uapi_i2c_master_read(I2C_MASTER_BUS_ID,

BH1750_ADDR >> 1, &data);

if (ret != 0) {

printf("BH1750:I2cRead(len:%d) failed, %0X!\n", data.receive_len,

ret);

return ;

}

bh1750_buff_H = data.receive_buf[0];

bh1750_buff_L = data.receive_buf[1];

}

3 完成CW2015的SDK的编写过程

该步骤主要包括如下部分。

(1) CW2015的“.h”文件

#include "pinctrl.h"

#include "common_def.h"

#include "soc_osal.h"

#include "gpio.h"

#include "hal_gpio.h"

#include "app_init.h"

#define CW2015_ADDR 0xC4

#define CW2015_REG_MODE 0x0A

#define CW2015_REG_VCELL_HIGH 0x02

#define CW2015_REG_VCELL_LOW 0x03

#define CW2015_MODE_AWAKE 0x00

void cw2015_init(void);

uint32_t cw2015_GetBatteryVoltage(void);

(2) CW2015的“.c”文件

#include "pinctrl.h"

#include "common_def.h"

#include "soc_osal.h"

#include "gpio.h"

#include "hal_gpio.h"

#include "app_init.h"

#include "cw2015.h"

#include "i2c.h"

#define I2C_MASTER_BUS_ID 1

uint32_t cw2015_vecll_buff_H = 0;

uint32_t cw2015_vecll_buff_L = 0;

三、实验步骤与操作方法

1 BH1750读取实验

研究BH1750的实现规则，完成函数

uint16_t bh1750_GetLightIntensity(void){

//中间部分需要用户完成

data = (bh1750_buff_H << 8) | bh1750_buff_L;

return (data * BH1750_RES)/1.2;

}

void bh1750_init(void){

//中间部分需要用户完成

osal_msleep(200);

osal_printk("BH1750 Init SUCC!\r\n");

}

2 CW2015实验

void cw2015_SendREG(uint8_t reg, uint8_t reg_data){

uint8_t buffer[] = {reg, reg_data};

i2c_data_t data = {0};

//中间部分需要用户完成

errcode_t ret = uapi_i2c_master_write(I2C_MASTER_BUS_ID,

CW2015_ADDR >> 1, &data);

if (ret != 0) {

printf("CW2015:I2cWriteREG(%02X) failed, %0X!\n", reg, ret);

return ;

}

}

uint8_t cw2015_ReadREG(uint8_t reg){

uint8_t send_buffer[] = {reg};

uint8_t read_buffer[1] = {0};

//中间部分需要用户完成

errcode_t ret = uapi_i2c_master_writeread(I2C_MASTER_BUS_ID,

CW2015_ADDR >> 1, &data);

if (ret != 0) {

printf("CW2015:I2cReadREG(%02X) failed, %0X!\n", reg, ret);

return 0;

}

return data.receive_buf[0];

}

uint32_t cw2015_GetBatteryVoltage(void){

uint32_t vol;

//中间部分需要用户完成

vol = ((cw2015_vecll_buff_H << 8) | cw2015_vecll_buff_L) * 305 / 1000;

return vol;

}

void cw2015_init(void){

cw2015_SendREG(CW2015_REG_MODE, CW2015_MODE_AWAKE);

osal_printk("CW2015 Init SUCC!\r\n");

}

四、实验结果与分析

1 实验项目“BH1750”的代码、测试过程和运行结果

2 实验项目“CW2015”的代码、测试过程和运行结果

五、讨论和建议